Workflow for microglial imaging…

Load in the Iba1 image. Adjust B/C so that they are probably slightly oversaturated. Apply.

Process -> Binary -> Make binary

Process -> Binary -> Dilate (do this 2x)

Process -> Binary -> Close

Analyze -> Analyze Particles -> Size = 1000-Infinity (only keeping big cells)

Check “Display results” and “Add to Manager”

Then open up your other phenotypic marker images like APOE, CD68, etc.

B/C doesn’t matter here. Select all of the ROI’s (shift, scroll to bottom, click)

Then click “Measure” and save the “Mean” results (MFI for each cell for that marker).

Save these in a CSV with a barcode number for the cell number, sample metadata, and marker (on top header row) with the expression data for each of the important markers.

These data are single-cell expression data. If the cell “barcode” is ‘Y_BCAS_1_cell#’  and you save the ROI measures for each sample you can go back to these and pick out individual cells and show images of these.

We can also calculate size of microglia and I think we can calculate the spine branch number and spine length etc if we use the skeletonize thing. All of that should be possible by using the ROI’s that are saved.